Wreath Hardware Instructions

REV 01 – 18 Dec 17

Female Connector

LED 0 is the one closest to the female connector

Holes for Hanging Wire

15

0

47

31

16

32

Backside View Shown – Insert the LED’s starting with #0 up to #47

- Cut off the female connector

- Strip and solder the 3 wires that went to the female connector to the Arduino Nano as follows:

  - Red – 5V

  - Blue – GND

  - White – D6

- Use the AC adaptor barrel adaptor and connect the wires as follows:

  - Red wire to + screw

  - Blue wire to – screw

- Cap off the two flying wires on the far end (end with male connector still on it) of the LED strand with heat shrink as they are not used.

- Glue the male connector at the far end to the backside of the wreath as it is not used.

Red Wire

Blue Wire

White Wire

Suggestions:

- Put a label next to the barrel connector that says ”5VDC Center + Only”

- Put a label on the AC adaptor that says it’s for the wreath